Lab #4 – Matt Cunningham

Digital Electronics

Combinational Logic Circuits

Objectives:

- Use Quartus to build a multilevel logic circuit using three inputs, three double-input AND gates, a triple-input OR gate, and an output LED.

- Determine the timing diagram based on the inputs and output given.

- Determine the truth table of possible outputs based on possible inputs by manipulating the switches on the board

Equipment Used:

- DE10 Development Board

- Quartus 18.1 Software

Procedure:

- Setup Quartus to create a new project

- Configure Quartus settings as required

- Design the logic circuit:

  - Add three inputs (A, B, C)

  - Add three double-input AND gates

  - Add a triple-input OR gate

  - Add a single output

  - Connect inputs C and B to AND0 (Becomes CB)

  - Connect inputs B and A to AND1 (Becomes BA)

  - Connect inputs A and C to AND2 (Becomes AC)

  - Connect CB, BA, and AC to the OR gate

  - Connect the OR gate to the output

- Run a simulation of the timing output

  - Set timing to 800 pico seconds

  - Set input C clock speed to 25

  - Set input B clock speed to 50

  - Keep input A clock speed at 100

- Connect DE10 board to system

- Use PIN planner to assign pins to our inputs

  - Input A = PIN_D12

  - Input B = PIN_C11

  - Input C = PIN_C10

  - LEDR0 = PIN_A8

- Test truth table logic by switching the applied input switches in the various configurations, to determine if the LED turns on appropriately based on the truth table output

Results:

Conclusion:

The circuit is working properly and I have demonstrated it to my professor. The truth table has been verified as working correctly based on all switch permutations. I learned how to effectively program the DE10 board